SHADOW RAM MEMORY CONTROLLERS

supported by

THE LAST BYTE MEMORY MANAGER (tm)

Version 2.52

Copyright (C) 1990-94

All Rights Reserved

by

KEY SOFTWARE PRODUCTS

440 Ninth Avenue

Menlo Park, California 94025-1802

415-364-9847

The Last Byte Memory Manager is a trademark of Key Software Products.

TABLE OF CONTENTS

CHAPTER 1 - INTRODUCTION        ...........................    1

1.1  Hardware Requirements       .......................    1

1.2  Checking Compatibility Using CHIPSET     ..........    1

CHAPTER 2 - SUPPORTED CHIPSETS       ......................    3

2.1  ACC Microelectronics       ........................    3

2.2  Acer Laboratories Incorporated (ALI)     ..........    3

2.3  AddTech Research (PCChip)      ....................    4

2.4  Appian Technology       ...........................    4

2.5  Atmel Corporation       ...........................    4

2.6  Biostar Systems (BIOTEQ)      .....................    5

2.7  Chips and Technologies      .......................    5

2.8  EFAR Microelectronics       .......................    6

2.9  ETEQ Microelectronics       .......................    6

2.10  Forex Computer Corporation      ..................    7

2.11  Headland Technology (G2)      ....................    7

2.12  Hedaka        ....................................    8

2.13  HINT Corporation       ...........................    8

2.14  Intel Corporation       ..........................    8

2.15  Macronix Incorporated (MX)      ..................    9

2.16  Micro Integration Corporation (MIC)     ..........    9

2.17  Mosel Corporation       ..........................    9

2.18  NEC Electronics       ............................   10

2.19  Oak Technology       .............................   10

2.20  OPTi Electronics       ...........................   10

2.21  PC Chip       ....................................   11

2.22  PicoPower Technology       .......................   11

2.23  Silicon Integrated Systems (SiS)     .............   12

2.24  Sun Electronics (SUNTAC)      ....................   12

2.25  Symphony Laboratories       ......................   13

2.26  Texas Instruments (TI)      ......................   13

2.27  UNIchip Incorporated       .......................   13

2.28  United Microelectronics Corporation (UMC)     ....   14

2.29  VIA Technologies       ...........................   14

2.30  Victronix Technology       .......................   14

2.31  VLSI Technology       ............................   15

CHAPTER 3 - UNSUPPORTED CHIPSETS       ....................   16

3.1  No Shadow Ram Support     .........................   16

3.2  Shadow Ram is not Read/Write    ...................   16

3.3  Overly Complex Control      .......................   16

3.4  Locked Configuration Registers      ...............   17

3.5  Configuration I/O Ports Reassigned     ............   17

Oct 09, 1994      SHADOW RAM MEMORY CONTROLLERS                 1

CHAPTER 1 - INTRODUCTION

1.1 Hardware Requirements

To  install The Last Byte Memory Manager, your computer must have

a  means  of providing read-write memory in the Upper Memory Area

between  640k  and  1 megabyte.  This is possible if you have any

of the following, listed in order of preference:

o  One  of  the  shadow  ram  memory  controller  chips

listed in this document, or

o  An  expanded memory board (EEMS, LIM 3.2 EMS, or LIM

4.0 EMS), or

�������������������������������������������������Ŀ

� NOTE: Using a LIM 3.2 EMS board is not advised  �

� as a method of providing upper memory.  It pro- �

� vides only 64k of upper memory and prevents ap- �

� plication software from using expanded memory.  �

���������������������������������������������������

o  Fixed  RAM memory already resident in the upper area

(such  as  the 32-bit "Built In Memory" installed in

some Compaq 386 machines), or

o  A  386sx,  386,  486,  or  486sx  processor  and the

Microsoft  Windows  3.0/3.1  or  DOS  5  versions of

EMM386.

1.2 Checking Compatibility Using CHIPSET

CHIPSET  is a program that attempts to determine if your computer

has  one  of the memory controllers required to use The Last Byte

Memory  Manager.   To  run  CHIPSET, simply enter its name on the

command line:

A>CHIPSET

and  follow  the  prompts.   CHIPSET  will  try  to  identify the

particular  memory controller you have, if any.  If it can't find

a  memory  controller, it will look for fixed memory in the upper

area  or  expanded  memory as a means of installing The Last Byte

Memory  Manager.   If  it  is  successful,  it  will give you the

option  of  automatically linking to the INSTALL program to setup

for use of The Last Byte Memory Manager.

CHIPSET   automatically  senses  whether  you  have  a  color  or

Copyright (C) 1990-94, Key Software Products. All Rights Reserved

Oct 09, 1994      SHADOW RAM MEMORY CONTROLLERS                 2

monochrome  display  to  determine  how  to  present  data on the

screen.   However,  you  may  force  the mode by using one of the

command  line  options:  /MONO  or  /COLOR (for monochrome liquid

crystal displays).

If  your  computer  has  one of the listed devices, then The Last

Byte Memory Manager should work on your computer.

Copyright (C) 1990-94, Key Software Products. All Rights Reserved

Oct 09, 1994      SHADOW RAM MEMORY CONTROLLERS                 3

CHAPTER 2 - SUPPORTED CHIPSETS

Key  Software  Products  is  continually  adding  support for new

chipsets  as  they  become  known to us and technical information

can  be  obtained.  Those that are currently supported in version

2.52  are  listed in the paragraphs below.  Some that we know of,

but  which  cannot  be  supported are listed near the end of this

document.

In  what  follows,  you will occassionally see "+32k" appended to

the  end  of  the  total  amount  of  shadow  ram indicated for a

particular  chipset.   This means that there is additional shadow

ram  in  the F000-FFFF region, but that it is read-only; however,

if  you  have a VGA or EGA adapter and a "split" bios, you may be

able  to  gain  up to an additional 32k by using the MOVE=OVERLAY

option of LASTBYTE.SYS.

An  adapter card (like some disk controllers) that uses a portion

of  the  upper  memory address space usually requires that shadow

ram  in  the  same address space be disabled.  Thus shadow ram is

usually  organized into segments that can be individually enabled

or  disabled.   Even  though  an  adapter  may  only  use a small

portion  of  one  shadow  ram segment, the entire segment must be

disabled.   How this affects your system can be determined by the

information under the columns labelled "Organization".

2.1 ACC Microelectronics

Part Number     Shadow Ram      Address Range   Organization

-----------     ----------      -------------   ------------

ACC2036         64k             Page Frame      4x16k

ACC2121         64k             Page Frame      4x16k

ACC2300         128k            E0000-FFFFF     1x128k

Part Number     Manufacturer's Description

-----------     --------------------------

ACC2036         Single Chip Solution for the 286 and 386SX

ACC2121         Turbo PC/AT Chip Set for the 286 and 386SX

ACC2300         82300 386 AT Chip Set

2.2 Acer Laboratories Incorporated (ALI)

Part Number     Shadow Ram      Address Range   Organization

-----------     ----------      -------------   ------------

M1207           384k            A0000-FFFFF     6x64k

M1209           128k            E0000-FFFFF     1x128k

M1217           256k            C0000-FFFFF     8x32k

M1219           256k            C0000-FFFFF     8x32k

Copyright (C) 1990-94, Key Software Products. All Rights Reserved

Oct 09, 1994      SHADOW RAM MEMORY CONTROLLERS                 4

M1409           256k            C0000-FFFFF     16x16k

M1419           256k            C0000-FFFFF     8x32k

M1429           256k            C0000-FFFFF     8x32k

Part Number     Manufacturer's Description

-----------     --------------------------

M1207           80286 Single Chip

M1209           80386SX PC/AT Super-Integration Chip

M1217           80386SX Single Chip

M1219           80486SX/80486DX Single Chip

M1409           80486SX/80486DX Single Chip

M1419           ISA 486/386 Cache System Chipset

M1429           486DX/P24T VESA Single-Chip

2.3 AddTech Research (PCChip)

AddTech  chipsets  are  usually  labelled  "PCChip"  or "Hedaka".

AddTech  designs  their  chips for use on their own motherboards,

and   they  identify  their  chipsets  by  the  motherboard  part

numbers.   There  is at least one AddTech chipset that is used on

two   different  motherboards  with  different  motherboard  part

numbers.   To  avoid  confusion,  The  Last  Byte  Memory Manager

refers  to  these  chips  using  "PCCHIP"  followed  by  a unique

number.

Part Number     Shadow Ram      Address Range   Organization

-----------     ----------      -------------   ------------

PCCHIP1         256k            C0000-FFFFF     16x16k

PCCHIP2         192k+32k        C0000-EFFFF     12x16k

Part Number     Manufacturer's Description

-----------     --------------------------

PCCHIP1         Used on 286/386SX AT motherboards M205 and M209

PCCHIP2         Used on 386 AT motherboard M321

2.4 Appian Technology

Part Number     Shadow Ram      Address Range   Organization

-----------     ----------      -------------   ------------

P915            256k            C0000-FFFFF     16x16k

Part Number     Manufacturer's Description

-----------     --------------------------

P915            486/386DX System Controller

Copyright (C) 1990-94, Key Software Products. All Rights Reserved

Oct 09, 1994      SHADOW RAM MEMORY CONTROLLERS                 5

2.5 Atmel Corporation

Part Number     Shadow Ram      Address Range   Organization

-----------     ----------      -------------   ------------

AT40281         192k+32k        C0000-EFFFF     12x16k

AT40283         192k+32k        C0000-EFFFF     12x16k

AT40391         192k+32k        C0000-EFFFF     12x16k

AT40410         256k            C0000-FFFFF     8x16k+2x64k

AT40493         192k+32k        C0000-EFFFF     12x16k

AT40495         192k+32k        C0000-EFFFF     12x16k

AT40498         192k+32k        C0000-EFFFF     12x16k

Part Number     Manufacturer's Description

-----------     --------------------------

AT40281         386SX Core Logic Controller w/Cache

AT40283         386SX Core Logic Controller w/o Cache

AT40391         80386DX PC/AT Chip Set

AT40410         ISA/PCI/VESA Chipset

AT40493         80486DX PC/AT Chip Set

AT40495         80486SX PC/AT Chip Set

AT40498         80486 Core Logic Controller

2.6 Biostar Systems (BIOTEQ)

Part Number     Shadow Ram      Address Range   Organization

-----------     ----------      -------------   ------------

82C3480         256k            C0000-FFFFF     8x32k

82C3491         256k            C0000-FFFFF     8x16k+2x64k

Part Number     Manufacturer's Description

-----------     --------------------------

82C3480         ISA 486/386 Cache System Chipset

82C3491         ISA 486/386 Single Chip

2.7 Chips and Technologies

Part Number     Shadow Ram      Address Range   Organization

-----------     ----------      -------------   ------------

82C100          64k             Page Frame      4x16k

82C212          384k            A0000-FFFFF     24x16k

82C222          384k            A0000-FFFFF     24x16k

82C230          384k            A0000-FFFFF     24x16k

82C235          384k            A0000-FFFFF     24x16k

82C241          384k            A0000-FFFFF     24x16k

82C302          384k            A0000-FFFFF     24x16k

82C307          384k            A0000-FFFFF     24x16k

82C311          384k            A0000-FFFFF     24x16k

82C322          384k            A0000-FFFFF     16x16k+1x128k

82C351          384k            A0000-FFFFF     24x16k

Copyright (C) 1990-94, Key Software Products. All Rights Reserved

Oct 09, 1994      SHADOW RAM MEMORY CONTROLLERS                 6

82C812          384k            A0000-FFFFF     24x16k

82C836          384k            A0000-FFFFF     24x16k

82C841          384k            A0000-FFFFF     24x16k

84021           384k            A0000-FFFFF     2x64k+4x16k+3x64k

84031           384k            A0000-FFFFF     2x64k+4x16k+3x64k

Part Number     Manufacturer's Description

-----------     --------------------------

82C100          IBM PS/2-30 and Super XT Compatible chip

82C212          CS8221 NEAT (tm) chipset

82C222          CHIPS/250 PS/2 50/60 chipset

82C230          PS/2 30 Single Chip Controller

82C235          SCAT (tm) Single Chip AT VLSI chip

82C241          CS8223 LeAPset (tm) chipset

82C302          CS8230 386/AT CHIPSet (tm)

82C307          CS8231 386/AT CHIPSet (tm)

82C311          CS8233 PEAK 386/AT CHIPSet (tm)

82C322          CHIPS/280 PS/2 70/80 chipset

82C351          CS82310 PEAK DM/386 AT CHIPSet (tm)

82C812          CS8281 NEATsx (tm) chipset

82C836          SCATsx (tm) Single Chip AT VLSI chip

82C841          CS8283 LeAPset-sx (tm) chipset

84021           ISA/486 chipset

84031           ISA/486 chipset

The  84021  provides  up  to 384k of memory, but any of the first

two  and  last  two  64k blocks might be "locked" and unavailable

due  to  the design of your particular bios.  This may reduce the

amount of memory from 384k to as little as 128k.

2.8 EFAR Microelectronics

Part Number     Shadow Ram      Address Range   Organization

-----------     ----------      -------------   ------------

82EC495         192k+32k        C0000-EFFFF     12x16k

82EC798         192k+32k        C0000-EFFFF     12x16k

EC802G          192k+32k        C0000-EFFFF     12x16k

Part Number     Manufacturer's Description

-----------     --------------------------

82EC495         EF8290WB 486/386 PC/AT ChipSet

82EC798         EC798 386/486 PC/AT Single Chip

EC802G          One Chip 32 Bits PC/AT Core Logic

2.9 ETEQ Microelectronics

Copyright (C) 1990-94, Key Software Products. All Rights Reserved

Oct 09, 1994      SHADOW RAM MEMORY CONTROLLERS                 7

Part Number     Shadow Ram      Address Range   Organization

-----------     ----------      -------------   ------------

ET6000          192k+32k        C0000-EFFFF     3x64k

ET82C390SX      192k+32k        C0000-EFFFF     12x16k

ET82C391        192k+32k        C0000-EFFFF     12x16k

ET82C491        192k+32k        C0000-EFFFF     12x16k

ET82C4901       192k+32k        C0000-EFFFF     12x16k

ET9000          192k+32k        C0000-EFFFF     3x64k

ET9800/ET391SX  192k+32k        C0000-EFFFF     3x64k

Part Number     Manufacturer's Description

-----------     --------------------------

ET6000          Cheetah 486DX/SX Non-Cache System

ET82C390SX      Panda 386SX Single Chip w/Direct Mapped Cache

ET82C391        Bobcat 386DX/486DX chipset

ET82C491        Cougar 386DX/486DX chipset

ET82C4901       Bengal 386DX/486DX chipset

ET9000          Jaguar 486DX/SX Cache System

ET9800/ET391SX  Firefox 386SX Write Back chipset

�������������������������������������������������Ŀ

� If you have a ET9800/ET391SX chipset and an     �

� adapter that occupies non-shadow ram address    �

� space between C8000-CFFFF, you will need to re- �

� configure that adapter to use another location. �

���������������������������������������������������

2.10 Forex Computer Corporation

Part Number     Shadow Ram      Address Range   Organization

-----------     ----------      -------------   ------------

FRX36C300       256k            C0000-FFFFF     6x32k+1x64k

FRX46C401       256k            C0000-FFFFF     6x32k+1x64k

FRX46C411       256k            C0000-FFFFF     6x32k+1x64k

FRX46C421       256k            C0000-FFFFF     6x32k+1x64k

FRX46C521       256k            C0000-FFFFF     6x32k+1x64k

FRX58C601       384k            A0000-FFFFF     128k+6x32k+64k

Part Number     Manufacturer's Description

-----------     --------------------------

FRX36C300       FRX36C300/200 Cache chipset

FRX46C401       FRX46C401/402 Cache chipset

FRX46C411       FRX46C411/412 Cache chipset

FRX46C421       FRX46C421/422 Cache chipset

FRX46C521       FRX46C521A Single Cache Chipset

FRX58C601       Pentium/P54C Cache Chipset

Copyright (C) 1990-94, Key Software Products. All Rights Reserved

Oct 09, 1994      SHADOW RAM MEMORY CONTROLLERS                 8

2.11 Headland Technology (G2)

Part Number     Shadow Ram      Address Range   Organization

-----------     ----------      -------------   ------------

HT12            64k             Page Frame      4x16k

HT18            128k            C0000-DFFFF     8x16k

HT21            128k            C0000-DFFFF     8x16k

HT22            128k            C0000-DFFFF     8x16k

HT103           128k            C0000-DFFFF     8x16k

HT113           128k            C0000-DFFFF     8x16k

Part Number     Manufacturer's Description

-----------     --------------------------

HT12            Single 286 AT Chip

HT18            286/386SX Advanced Memory Manager Chip

HT21            286/386SX Advanced Memory Manager Chip

HT22            286/386SX Advanced Memory Manager Chip

HT103           286/386SX Advanced Memory Manager Chip

HT113           286/386SX Advanced Memory Manager Chip

2.12 Hedaka

See section on AddTech Research.

2.13 HINT Corporation

Part Number     Shadow Ram      Address Range   Organization

-----------     ----------      -------------   ------------

HINT8001        256k            C0000-FFFFF     4x32k+2x64k

HINT8005        256k            C0000-FFFFF     4x32k+2x64k

Part Number     Manufacturer's Description

-----------     --------------------------

HINT8001        Caesar Super ISA chipset

HINT8005        VLISA Single Chip

2.14 Intel Corporation

Part Number     Shadow Ram      Address Range   Organization

-----------     ----------      -------------   ------------

386SL           384k            A0000-FFFFF     24x16k

486SL           384k            A0000-FFFFF     24x16k

486SLz          384k            A0000-FFFFF     24x16k

82343           192k+32k        C0000-EFFFF     12x16k

82346           192k+32k        C0000-EFFFF     12x16k

82359           384k            A0000-FFFFF     24x16k

82424           256k            C0000-FFFFF     12x16k+64k

82434           256k            C0000-FFFFF     12x16k+64k

Copyright (C) 1990-94, Key Software Products. All Rights Reserved

Oct 09, 1994      SHADOW RAM MEMORY CONTROLLERS                 9

Part Number     Manufacturer's Description

-----------     --------------------------

386SL           i386SL Microprocessor

486SL           i386SL Microprocessor

486SLz          i486SLz Microprocessor (no coprocessor)

82343           82340SX Chip Set

82346           82340DX Chip Set

82359           82350DT EISA Chip Set

82424           82420 PCIset (Aries, Saturn I, Saturn II)

82434           82430 PCIset (Mercury, Neptune)

2.15 Macronix Incorporated (MX)

Part Number     Shadow Ram      Address Range   Organization

-----------     ----------      -------------   ------------

MX83C305        256k            C0000-FFFFF     6x32k+1x64k

Part Number     Manufacturer's Description

-----------     --------------------------

MX83C305        Compactest 386DX Chipset with Cache

2.16 Micro Integration Corporation (MIC)

Part Number     Shadow Ram      Address Range   Organization

-----------     ----------      -------------   ------------

MIC9283         192k+32k        C0000-EFFFF     12x16k

MIC9382         192k+32k        C0000-EFFFF     12x16k

MIC9391         192k+32k        C0000-EFFFF     12x16k

MIC9498         192k+32k        C0000-EFFFF     12x16k

Part Number     Manufacturer's Description

-----------     --------------------------

MIC9283         386SX PC/AT Single Chip w/Posted Write or Write-Thru

MIC9382         386DX PC/AT Chip Set w/Write-Thru Cache

MIC9391         386DX PC/AT Chip Set w/Write-Back Cache

MIC9498         486DX PC/AT Single Chip w/Write-Back Cache

2.17 Mosel Corporation

Part Number     Shadow Ram      Address Range   Organization

-----------     ----------      -------------   ------------

MS400           256k            C0000-FFFFF     4x16k+3x64k

Part Number     Manufacturer's Description

-----------     --------------------------

MS400           486SX/DX Single Chip AT

Copyright (C) 1990-94, Key Software Products. All Rights Reserved

Oct 09, 1994      SHADOW RAM MEMORY CONTROLLERS                10

2.18 NEC Electronics

Part Number     Shadow Ram      Address Range   Organization

-----------     ----------      -------------   ------------

PWRMATE286      352k            A0000-FFFFF     2x64k+32k+3*64k

This chip has no shadow ram at C8000-CFFFF.

Part Number     Manufacturer's Description

-----------     --------------------------

PWRMATE286      NEC Powermate 286 Plus Notebook

2.19 Oak Technology

Part Number     Shadow Ram      Address Range   Organization

-----------     ----------      -------------   ------------

OTI-021         256k            C0000-FFFFF     4x32k+2x64k

OTI-041         256k            C0000-FFFFF     4x32k+2x64k

Part Number     Manufacturer's Description

-----------     --------------------------

OTI-021         OTI-020 Desktop PC Chip Set

OTI-041         OTI-040 OakNote Chip Set

2.20 OPTi Electronics

Part Number     Shadow Ram      Address Range   Organization

-----------     ----------      -------------   ------------

82C281          192k+32k        C0000-EFFFF     12x16k

82C282          192k+32k        C0000-EFFFF     12x16k

82C283          192k+32k        C0000-EFFFF     12x16k

82C291          256k            C0000-FFFFF     12x16k+1x64k

82C295          256k            C0000-FFFFF     12x16k+1x64k

82C382          192k+32k        C0000-EFFFF     12x16k

82C391          192k+32k        C0000-EFFFF     12x16k

82C461          192k+32k        C0000-EFFFF     12x16k

82C463          192k+32k        C0000-EFFFF     12x16k

82C482          192k+32k        C0000-EFFFF     12x16k

82C491          192k+32k        C0000-EFFFF     12x16k

82C493          192k+32k        C0000-EFFFF     12x16k

82C495          192k+32k        C0000-EFFFF     12x16k

82C495SLC       192k+32k        C0000-EFFFF     12x16k

82C495SX        192k+32k        C0000-EFFFF     12x16k

82C495XLC       192k+32k        C0000-EFFFF     12x16k

82C496A         192k+32k        C0000-EFFFF     12x16k

82C496B         256k            C0000-FFFFF     12x16k+1x64k

82C498          192k+32k        C0000-EFFFF     12x16k

82C499          192k+32k        C0000-EFFFF     12x16k

Copyright (C) 1990-94, Key Software Products. All Rights Reserved

Oct 09, 1994      SHADOW RAM MEMORY CONTROLLERS                11

82C547          256k            C0000-FFFFF     8x16k+2x64k

82C597          256k            C0000-FFFFF     8x16k+2x64k

82C682          256k            C0000-FFFFF     8x16k+2x64k

82C683          256k            C0000-FFFFF     8x16k+2x64k

82C691          256k            C0000-FFFFF     8x16k+2x64k

82C693          256k            C0000-FFFFF     8x16k+2x64k

82C801          192k+32k        C0000-EFFFF     12x16k

82C802          192k+32k        C0000-EFFFF     12x16k

82C895          192k+32k        C0000-EFFFF     12x16k

Part Number     Manufacturer's Description

-----------     --------------------------

82C281          Cache SX/AT chipset

82C282          Non-Cache SX/AT chipset

82C283          Block Interleave SX/AT chipset

82C291          SXWB PC/AT chipset

82C295          SLCWB PC/AT chipset

82C382          HiD/386 AT chipset

82C391          386WB PC/AT chipset

82C461          Notebook PC/AT chipset

82C463          SCNB Single Chip Notebook chipset

82C482          HiB/486 AT chipset

82C491          486SXWB PC/AT chipset

82C493          486SXWB PC/AT chipset

82C495          486SXWB PC/AT chipset

82C495SLC       495SLC PC/AT chipset

82C495SX        LCWB PC/AT chipset

82C495XLC       PC/AT chipset

82C496A         DXBB PC/AT chipset (Rev. A)

82C496B         DXBB PC/AT chipset (Rev. B)

82C498          DXWB PC/AT chipset

82C499          DXSC PC/AT Single Chip

82C547          Python Chipset for Pentium

82C597          PTMA WB-V PC/AT chipset

82C682          386/486WB EISA chipset

82C683          386/486AWB EISA chipset

82C691          PC/AT EISA-LCWB Chipset

82C693          Pentium EISA Chipset

82C801          SCWB2 PC/AT Single Chip

82C802          SCWB2 PC/AT Single Chip

82C895          System/Power Management Controller

2.21 PC Chip

See section on AddTech Research.

Copyright (C) 1990-94, Key Software Products. All Rights Reserved

Oct 09, 1994      SHADOW RAM MEMORY CONTROLLERS                12

2.22 PicoPower Technology

Part Number     Shadow Ram      Address Range   Organization

-----------     ----------      -------------   ------------

PT86C168        256k            C0000-FFFFF     12x16k+1x64k

PT86C268        256k            C0000-FFFFF     12x16k+1x64k

Part Number     Manufacturer's Description

-----------     --------------------------

PT86C168        Evergreen Portable Computer Core Logic Chip

PT86C268        Evergreen HV Core Logic Chip (Hybrid 3.3v/5.0v)

2.23 Silicon Integrated Systems (SiS)

Part Number     Shadow Ram      Address Range   Organization

-----------     ----------      -------------   ------------

85C360          256k            C0000-FFFFF     6x32k+1x64k

85C401          256k            C0000-FFFFF     6x32k+1x64k

85C411          256k            C0000-FFFFF     6x32k+1x64k

85C460          256k            C0000-FFFFF     6x32k+1x64k

85C461          256k            C0000-FFFFF     6x32k+1x64k

85C471          256k            C0000-FFFFF     6x32k+1x64k

Part Number     Manufacturer's Description

-----------     --------------------------

85C360          ISA 386DX Single Chip

85C401          ISA 486DX/SX Chipset

85C411          EISA 386/486 Chipset

85C460          ISA 486DX/SX Single Chip

85C461          ISA 486DX/SX Single Chip

85C471          Green PC ISA-VESA Single Chip

2.24 Sun Electronics (SUNTAC)

All  SunTac  chips  provide  128k  of  memory  in four 32k blocks

between  E0000-FFFFF.   An  additional  128k  in eight 16k blocks

between  C0000-DFFFF  is available at the expense of all Expanded

Memory  by adding the word NOEMS to the SUNTAC Physical option as

in: Physical=ST62C202,NOEMS.

Part Number     Shadow Ram      Address Range   Organization

-----------     ----------      -------------   ------------

ST62C202        128k/256k       C0000-FFFFF     [8x16k]+4x32k

ST62C241        128k/256k       C0000-FFFFF     [8x16k]+4x32k

ST62C251        128k/256k       C0000-FFFFF     [8x16k]+4x32k

Copyright (C) 1990-94, Key Software Products. All Rights Reserved

Oct 09, 1994      SHADOW RAM MEMORY CONTROLLERS                13

Part Number     Manufacturer's Description

-----------     --------------------------

ST62C202        Chipset 20 for 286

ST62C241        Chipset 24 for 386

ST62C251        Chipset 25 for 286 and 386sx

2.25 Symphony Laboratories

Part Number     Shadow Ram      Address Range   Organization

-----------     ----------      -------------   ------------

SL82C361        256k            C0000-FFFFF     8x16k+2x64k

SL82C461        256k            C0000-FFFFF     8x16k+2x64k

SL82C471        256k            C0000-FFFFF     8x16k+2x64k

Part Number     Manufacturer's Description

-----------     --------------------------

SL82C361        Haydn SL82C360 80386DX/SX chipset

SL82C461        Haydn SL82C460 80486 chipset

SL82C471        Mozart SL82C470 486/386 EISA chipset

2.26 Texas Instruments (TI)

Part Number     Shadow Ram      Address Range   Organization

-----------     ----------      -------------   ------------

TACT82411       384k            A0000-FFFFF     24x16k

TACT82S411      384k            A0000-FFFFF     24x16k

TACT83442       256k            C0000-FFFFF     16x16k

Part Number     Manufacturer's Description

-----------     --------------------------

TACT82411       Snake Single-Chip AT Controller

TACT82S411      Snake+ Single-Chip AT Controller

TACT83442       Tiger TACT83000 AT Chip Set

�������������������������������������������������Ŀ

� The TACT83442 can only be supported if the main �

� bios has not reassigned the I/O ports that are  �

� used to access the configuration registers from �

� their default values.                           �

���������������������������������������������������

2.27 UNIchip Incorporated

Part Number     Shadow Ram      Address Range   Organization

-----------     ----------      -------------   ------------

U4800           192k+32k        C0000-EFFFF     3x64k

U4800VLX        192k+32k        C0000-EFFFF     12x16k

Copyright (C) 1990-94, Key Software Products. All Rights Reserved

Oct 09, 1994      SHADOW RAM MEMORY CONTROLLERS                14

Part Number     Manufacturer's Description

-----------     --------------------------

U4800           386/486 AT Chip Set

U4800VLX        386/486 AT Chip Set

2.28 United Microelectronics Corporation (UMC)

Part Number     Shadow Ram      Address Range   Organization

-----------     ----------      -------------   ------------

UM82C212        384k            A0000-FFFFF     24x16k

UM82C391        256k            C0000-FFFFF     8x16k+2x64k

UM82C481        256k            C0000-FFFFF     8x16k+2x64k

UM82C491        256k            C0000-FFFFF     8x16k+2x64k

Part Number     Manufacturer's Description

-----------     --------------------------

UM82C212        UM82C210 386SX/286 AT Chip Set

UM82C391        UM82C390 40Mhz 80386 Chip Set

UM82C481        UM82C480 50Mhz 80486 Chip Set

UM82C491        UM82C491 386/486 Single Chip

2.29 VIA Technologies

Part Number     Shadow Ram      Address Range   Organization

-----------     ----------      -------------   ------------

VT82C470        256k            C0000-FFFFF     8x16k+2x64k

VT82C475        256k            C0000-FFFFF     8x16k+2x64k

VT82C486        256k            C0000-FFFFF     8x16k+2x64k

VT82C495        256k            C0000-FFFFF     8x16k+2x64k

VT82C496        256k            C0000-FFFFF     8x16k+2x64k

Part Number     Manufacturer's Description

-----------     --------------------------

VT82C470 - From the Jupiter Chip Set (w/o cache)

VT82C475 - From the Jupiter Chip Set (w/cache)

VT82C486 - From the VL/ISA Green PC Solution

VT82C495 - From the Venus Chip Set

VT82C496 - From the Green PC 80486 PCI/VL/ISA System

2.30 Victronix Technology

Part Number     Shadow Ram      Address Range   Organization

-----------     ----------      -------------   ------------

VIC92C4800      192k+32k        C0000-EFFFF     12x16k

Copyright (C) 1990-94, Key Software Products. All Rights Reserved

Oct 09, 1994      SHADOW RAM MEMORY CONTROLLERS                15

Part Number     Manufacturer's Description

-----------     --------------------------

VIC92C4800      486DX/SX ISA PC/AT Non-Cache Single Chip

2.31 VLSI Technology

Part Number     Shadow Ram      Address Range   Organization

-----------     ----------      -------------   ------------

VL82C202        384k            A0000-FFFFF     6x64k

VL82C310        384k            A0000-FFFFF     4x32k+8x16k+4x32k

VL82C311        384k            A0000-FFFFF     4x32k+8x16k+4x32k

VL82C315        384k            A0000-FFFFF     4x32k+8x16k+4x32k

VL82C316        384k            A0000-FFFFF     4x32k+8x16k+4x32k

VL82C320        192k+32k        C0000-EFFFF     12x16k

VL82C330        192k+32k        C0000-EFFFF     12x16k

VL82C480        384k            A0000-FFFFF     24x16k

VL82C481        384k            A0000-FFFFF     24x16k

VL82C486        384k            A0000-FFFFF     24x16k

Part Number     Manufacturer's Description

-----------     --------------------------

VL82C202        PC/AT-Compatible High Speed Memory Controller

VL82C310        SCAMP 286/386sx Chip Set

VL82C311        SCAMP 286/386sx Chip Set

VL82C315        SCAMP II PC/AT-Compatible System Controller

VL82C316        SCAMP II Chip Set for Power-Managed Portables

VL82C320        TOPCAT 286/386sx Chip Set

VL82C330        TOPCAT 386 DX Chip Set

VL82C480        486 System/Cache/ISA Bus Controller

VL82C481        486 System/Cache/ISA Bus Controller

VL82C486        Single Chip, High Performance 486 PC/AT Controller

�������������������������������������������������Ŀ

� Pin 69 on th VL82C202 controls whether its      �

� shadow ram is enabled or relocated.  This pin   �

� is usually tied to a jumper on the motherboard  �

� which must be set properly to work.             �

���������������������������������������������������

Copyright (C) 1990-94, Key Software Products. All Rights Reserved

Oct 09, 1994      SHADOW RAM MEMORY CONTROLLERS                16

CHAPTER 3 - UNSUPPORTED CHIPSETS

If  your computer has shadow ram capability, then it probably has

a  shadow ram memory controller.  However, some of these chipsets

cannot  be  supported by The Last Byte Memory Manager for reasons

shown below.

Some  of  these  chips, however, do have hardware EMS support and

thus  many  of  their motherboards come with an EMS device driver

designed  specifically  to take advantage of this capability.  So

in  many  cases,  you can install this driver first, and then The

Last  Byte  Memory  Manager can use it to manage the upper memory

area.

3.1 No Shadow Ram Support

Some   of   the   older   chipsets  do  not  provide  shadow  ram

capability.

Examples: Western Digital chipsets

Texas Instruments TACT82300 series

Early Suntac chipsets (ST62BC001-6)

United Microelectronics Corp. UM82C231

3.2 Shadow Ram is not Read/Write

Some  chipsets  have  shadow ram, but can only put it into either

write-only  mode  (for copying rom contents to ram), or read-only

mode  (to  prevent accidental writes into what's supposed to be a

copy  of  rom).  The hardware EMS in some such chipsets sometimes

provides  a limited amount (e.g., 64k) of Hi-DOS memory, but only

if the user doesn't need expanded memory.

Examples: CONTAQ 82C591 (no hardware EMS support)

Headland/G2 (many do have hardware EMS however)

ACC Microelectronics (many do have hardware EMS)

USA Integration (USAi) (no hardware EMS)

3.3 Overly Complex Control

The  Last Byte Memory Manager software is table-driven, making it

relatively   easy   to   adapt  to  different  memory  controller

architectures, but there are limits!

Examples: Zymos/Appian System 90 chipset

United Microelectronics Corp UM82C384, '388, '389

Forex Computer Corp. FRX36C100

Copyright (C) 1990-94, Key Software Products. All Rights Reserved

Oct 09, 1994      SHADOW RAM MEMORY CONTROLLERS                17

Silicon Integrated System (SiS) 85C310.

3.4 Locked Configuration Registers

Some  memory  controllers  are  designed  so  that  they  can  be

"locked"  at the end of the main bios initialization, requiring a

hardware  reset  to  unlock,  and  thus  preventing The Last Byte

Memory Manager from accessing the controller.

Examples: Intel 82335, 82335SX

3.5 Configuration I/O Ports Reassigned

Some  memory  controllers  are  designed so that the "index/data"

pair  of I/O ports used to access the configuration registers can

be  reassigned  to  different  I/O  addresses.   This  makes  the

configuration  registers  effectively  inaccessible  to  The Last

Byte  Memory  Manager since it has no way of knowing what new I/O

port addresses may have been established within the main bios.

Examples: Texas Instruments TACT84500

GS Technology GS62CS03

Copyright (C) 1990-94, Key Software Products. All Rights Reserved

#